File:          ltuvi_update.doc

Last updated:  20 May 2008

# LTUvi - Software Model Update

# Table of contents

Page

The VME address map of the LTUvi board			  2

References								  3

Introduction								  4

M	Modification of the existing words				  4

M.1	BC_STATUS word						  4

M.2	TTC interface word						  4

TTC_INTERFACE word					  4

M.3	SnapShot memory Command and Status words		  5

SSM_COMMAND word					  5

SSM_STATUS word						  5

M.3.1	SnapShot memory data bit allocation				  6

R	References for some of new words				  6

R.1	Software LED word						  6

SOFT_LED word						  6

R.2	BusyProbe option						  6

MINIMAX_SELECT word					  7

R.3	LTUvi counters						  7

R.3.1	List of LTUvi counters					  7

Time counters						  7

Fast-signal counters					  8

Slow-signal counters					  8

N	Definition of new LTUvi words					  8

N.1	Reset FPGA Phase Locked Loop command			  8

PLL_RESET word						  8

N.2	TTCvi emulation logic					  8

N.2.1	General							  8

N.2.2	Transmision of ORBIT and PREPULSE			  9

ORBIT_TIME word						10

PP_TIME word						10

N.2.3	TTC data transmission from the VME processor		10

TTC_DATA word						10

TTC_STATUS word						11

N.3	ScopeProbe signal selection - LTUvi board			12

N.4	Front panel LEDs - update for the LTUvi board		14

N.5	Front panel connections - update for the LTUvi board	14

N.6	Cable connections in the TTC partition- LTUvi board	14

# The VME address map of the LTUvi board

| Address | Mnemonic | Description | Wr | Rd | Section |
| --- | --- | --- | --- | --- | --- |
|  |  | VME Controller FPGA |  |  |  |
| H”01” | CODE_ADD | LTU Board Type code |  | + | 3.2.1 |
| H”02” | SERIAL_NUMBER | LTU board Serial Number |  | + | 3.2.2 |
| H”03” | VERSION_ADD | VME controller FPGA Firmware Version |  | + | 3.2.3 |
| H”0A” | SOFT_RESET | LTU software reset (dummy data) | + |  | 3.2.1 |
| H”10” | FLASHACCESS_INCR | Flash memory access with add. increment | + | + | 3.5 |
| H”11” | FLASHACCESS_NOINCR | Flash memory access, no add. increment | + | + | 3.5 |
| H”12” | FLASHADD_CLEAR | Clear Flash Memory Address | + |  | 3.5 |
| H”13” | FLASH_STATUS | Flash memory Status word |  | + | 3.5 |
| H”14” | CONFIG_STATUS | Configuration Status word |  | + | 3.6.1 |
| H”15” | CONFIG_START | Start Configuration command | + |  | 3.6.1 |
| H”16” | TEMP_START | Start Temperature Read command | + |  | 3.6.2 |
| H”17” | TEMP_STATUS | Temperature Read Status |  | + | 3.6.2 |
| H”18” | TEMP_READ | Temperature Read Data |  | + | 3.6.2 |
| H”19” | MASTER_MODE | Master write destination (TTCvi or LTU) | + | + | 3.6.3 |
|  |  | LTU Logic FPGA |  |  |  |
| H”20” | VERSION_ADD | LTU logic FPGA Firmware Version |  | + | 3.2.3 |
| H”31” | BC_STATUS | BC Status word |  | + | M.1 |
| H”33” | ADC_START | ADC Start command (dummy data) | + |  | 3.7.1 |
| H”34” | ADC_DATA | ADC Data (including ADC BUSY status) |  | + | 3.7.1 |
| H”35” | ERROR_ENABLE | Enable Error Emulation command | + |  | 3.15 |
| H”36” | ERROR_STATUS | Error Emulation Status word |  | + | 3.15 |
| H”37” | ERROR_SELECTOR | Error selector (Set Error mode) | + |  | 3.15 |
| H”38” | ERROR_DEMAND | Error on Demand request | + |  | 3.15 |
| H”3A” | ORBIT_CLEAR | Clear emulator’s ORBIT counter | + |  | 3.13 |
| H”3B” | EMULATION_START | Start Emulation command (dummy data) | + |  | 3.13 |
| H”3C” | QUIT_SET | Set QUIT command (dummy data) | + |  | 3.13 |
| H”3D” | BREAK_SET | Set BREAK command (dummy data) | + |  | 3.13 |
| H”45” | SLM_ADD_CLEAR | Clear SLM Read and Write Addresses | + |  | 3.13 |
| H”46” | SLM_DATA | SLM Data word |  | + | 3.13 |
| H”47” | EMU_STATUS | Emulator BUSY status |  | + | 3.13 |
| H”49” | SOFT_TRIGGER | Software generated START | + |  | 3.14 |
| H”4E” | BUSY_ENABLE | BUSY 1/BUSY2 input enable | + |  | 3.10 |
| H”4F” | BYSY_STATUS | BUSY status word |  | + | 3.10 |
| H”50” | SOFT_BUSY | Software generated BUSY | + |  | 3.10 |
| H”52” | L1MAX_CLEAR | Clear L1 FIFO Maximum Occupancy | + |  | 3.11 |
| H”53” | L2MAX_CLEAR | Clear L2 FIFO Maximum Occupancy | + |  | 3.11 |
| H”56” | FIFO_MAX | L1/L2 FIFO Maximum Occupancy |  | + | 3.11 |
| H”57” | SOFT_LED | SoftLED option read word |  | + | R.1 |
| H”5A” | BUSYMAX_DATA | BusyProbe option - longest BUSY time |  | + | R.2 |
| H”5B” | BUSYMINI_DATA | BusyProbe option - shortest BUSY time |  | + | R.2 |
| H”5D” | MINIMAX_CLEAR | Clear BUSYMAX/BUSYMINI counters | + |  | R.2 |
| H”66” | PIPELINE_CLEAR | Pipeline Clear command (dummy data) | + |  | 3.13 |
| H”67” | SSM_COMMAND | SSM Command word | + |  | M.3 |
| H”68” | SSM_START | SSM Start command (dummy data) | + |  | 3.17 |
| H”69” | SSM_STOP | SSM Stop command (dummy data) | + |  | 3.17 |
| H”6A” | SSM_ADDRESS | SSM Address word | + | + | 3.17 |
| H”6B” | SSM_DATA | SSM Data word | + | + | 3.17 |
| H”6C” | SSM_STATUS | SSM Status word |  | + | M.3 |
| H”6E” | TIMING_TEST | Timing Test word | + |  | 3.19 |
| H”6F” | PLL_RESET | PLL Reset command (dummy data) | + |  | N.1 |
| H”72” | TTC_STATUS | TTC FIFO status word |  | + | N.2 |
| H”75” | COPY_COUNT | Copy counter command (dummy data) | + |  | R.3 |
| H”76” | COPY_BUSY | Copy BUSY status word |  | + | R.3 |
| H”77” | COPY_CLEARADD | Clear copy memory add. (dummy data) | + |  | R.3 |
| H”78” | COPY_READ | Copy memory data read |  | + | R.3 |
| H”132” | BC_DELAY_ADD | BC delay register (5 bits, 1ns/bit) | + | + | 3.7.2 |
| H”139” | ERROR_RATE | Error rate setting | + | + | 3.15 |
| H”13E” | LAST_BC | Number of the last BC in the LHC orbit | + | + | 3.8 |
| H”13F” | PREPULSE_BC | BC number of the PREPULSE | + | + | 3.8 |
| H”140” | CALIBRATION_BC | BC number of the calibration trigger | + | + | 3.8 |
| H”141” | GAP_BC | First BC of the LHC Large Gap | + | + | 3.8 |
| H”142” | ORBIT_BC | BC number of the emulator’s ORBIT | + | + | 3.8 |
| H”143” | L1_DELAY | Emulator’s L0 to L1 delay (in BCs) | + | + | 3.8 |
| H”144” | L2_DELAY | Emulator’s L0 to L2 delay (in BCs) | + | + | 3.8 |
| H”148” | START_SET | Settings for the START signal | + | + | 3.14 |
| H”14A” | RANDOM_NUMBER | Rate of the Random START | + | + | 3.14 |
| H”14B” | COUNT_PERIOD | Period of the Scaled-down BC START | + | + | 3.14 |
| H”14C” | TTC_INTERFACE | Channel A/B output delay; L0 over TTC | + | + | M.2 |
| H”14D” | MODE | Setting of Global/Stand-alone mode | + | + | 3.22 |
| H”151” | L1_FORMAT | Setting of the L1 Message format | + | + | 3.23 |
| H”154” | SCOPE_SELECT | Selection of ScopeProbe A/B signals | + | + | 3.18 |
| H”155” | BACKPLANE_EN | Backplane output enable | + | + | 3.12 |
| H”15C” | MINIMAX_SELECT | Select input for MinimaxBusy option | + | + | R.2 |
| H”15E” | MINIMAX_LIMIT | Threshold for LongBusy option | + | + | R.2 |
| H”16B” | CLEAR_COUNTER | Clear counter command | + | + | R.3 |
| H”170” | ORBIT_TIME | Start of the ORBIT transmission | + | + | N.2 |
| H”171” | PP_TIME | Start of the PREPULSE transmission | + | + | N.2 |
| H”172” | TTC_DATA | TTC FIFO write data | + | + | N.2 |
| H”1FA” | TEST_ADD | LED Test flag | + | + | 3.3.1 |

Address is the local address (see section 3.1.6).

Note:	Write and read addresses H"1NN" are read from the SpyMemory - the read data are correct only after the preceding write.

Mnemonic is the symbolic name used in the LTU FPGA firmware documentation.

Wr and Rd refer to the VME write and VME read access.

A very brief Description is expanded in the corresponding section of [1], for pointers of the type 3.7.2 and similar; or in the following sections M.1-3, R.1-3 and N.1-2.

References:	[1]  LTU Software Model, [6]

[2]  [6], CTP Documentation, List of Software Notes

[3]  TTCrx Reference Manual, [7]

[4]  TTCvi - MkII, TTC-VMEbus Interface, [7]

[5]  LTU Preliminary Design Review, [6]

[6]  ALICE Trigger web site: http://epweb2.ph.bham.ac.uk/user/pedja/alice/

[7]  TTC web site: http://ttc.web.cern.ch/ttc/

## Introduction

The LTUvi board - the original LTU board with the version H"B0" or later of the LTU Logic FPGA firmware - performs all the logic functions required in the ALICE TTC partition; the TTCvi board is not required any longer.

The BusyProbe option has been added to monitor and analyse elements of sub-detector BUSY; the list of internal signals accessible via the ScopeProbe option has been modified, as well as the bit-allocation in the SnapShot memory; some new control and status words have been added; some "old" words have been either modified or removed. All the modification and additions have been documented in this note.

LTU Software Model [1] still remains the main software reference, but the updated VME Address Map of the LTUvi Board should be consulted first: it contains pointers (column Section) to the up-to-date documentation.

## M	Modifications of the existing words

### M.1	BC Status word

BC_STATUS word, read only:

Address: H"31"

Bit allocation:

Data bits		Content

D[0]	       	      BC Error flag

D[1]	       	      PLL Locked flag

D[2]	       	      ORBIT Error flag

The ORBIT Error flag indicates either the absence or a wrong (too fast or too slow) frequency of the ORBIT input signal; see [1], section 3.20 for the rest of application details.

### M.2	TTC Interface word

TTC_INTERFACE word, write and read:

Address: H"14C"

Bit allocation:

Data bits		Content

D[0]		Delay Channel B flag

D[1]		Delay Channel A flag

D[3]		Enable LO over TTC option

When the Delay flag is asserted, the corresponding Channel A/Channel B output (LTU's connector L1/ORBIT) is delayed by a half of a BC interval (12.5ns). The option is provided in order to comply with the TTCex board set-up time and hold time requirements.

The L0 over TTC option is active when the Enable bit is asserted.

Note:	Data bit D[2] is not used.

See [1], section 3.9 for the rest of application details.

### M.3	SnapShot memory Command and Status words

SSM_COMMAND word, write only:

Address: H"67"

Bit allocation:

Data bits		Content

D[4]		Select CTP flag

D[1]		Operation bit

D[0]		Mode bit

If the Select CTP flag is asserted, the CTP inputs to the LTU are connected to the data bits D[0..6] of the SnapShot memory, regardless of the state of the Mode bit; the option makes the LTU compatible with all the other trigger boards for tests of connectivity. Use of the Operation and the Mode bits is explained in [1], section 3.17.

SSM_STATUS word, read only:

Address: H"6C"

Bit allocation:

Data bits		Content

D[4]		Select CTP flag

D[2]		SSM BUSY

D[1]		Operation bit

D[0]		Mode bit

The Mode, the Operation, and the Select CTP flags are set by the SSM_COMMAND word; the SSM BUSY flag is controlled by the hardware.

### M.3.1	SnapShot memory data bit allocation

| Data | Firmware notation | Signal | Description |
| --- | --- | --- | --- |
| 0 | !select_ctp & ltu_select.orbit_sel # select_ctp & !/ctp_orbit | ORBIT | Selected ORBIT (1µs pulse) |
| 1 | !select_ctp & ltu_select.prepulse # select_ctp & !/ctp_prepulse | PP request | PP request (25ns pulse); includes errors |
| 2 | !select_ctp & ssh_l0 # select_ctp & !/ctp_l0 | L0 output | L0 output; includes errors |
| 3 | !select_ctp & ltu_select.l1_strobe # select_ctp & !/ctp_l1 | L1 STROBE | Selected L1 STROBE; includes errors |
| 4 | !select_ctp & ltu_select.l1_data # select_ctp & !/ctp_l1_data | L1 DATA | Selected L1 DATA |
| 5 | !select_ctp & ltu_select.l2_strobe # select_ctp & !/ctp_l2_strobe | L2 STROBE | Selected L2 STROBE; includes errors |
| 6 | !select_ctp & ltu_select.l2_data # select_ctp & !/ctp_l2_data | L2 DATA | Selected L2 DATA |
| 7 | ltu_busy.s_sbusy | BUSY1+BUSY2 | Subdetector BUSY (BUSY1 + BUSY2) |
| 8 | ltu_busy.s_busy | BUSY | LTU internal BUSY; all constituents |
| 9 | l1fifo_wr.l1nearly_full | L1FIFO full | L1 Message FIFO nearly full status |
| 10 | l2fifo_wr.l2nearly_full | L2FIFO full | L2 Message FIFO nearly full status |
| 11 | out_l1 | Channel A | TTC Channel A output |
| 12 | out_orbit | Channel B | TTC Channel B output |
| 13 | ttcvi.ttc_busy | TTC BUSY | TTC Channel B transmission in progress |
| 14 | ttcvi.pp_select | PP transmit | PP transmission in progress (17 BCs) |
| 15 | strobe | VME strobe | VME strobe (read or write) |
| 16 | ssh_start_all.q | START | All emulation START trigger options |
| 17 | ltu_select.any_error | Any error | Any error signal (25ns pulse) |

Note:	The Select CTP flag is set with the SSM_COMMAND word (see above).

## R	References for some of the new words

Some of the logic blocks have been used on other trigger boards; the corresponding control words have been documented elsewhere. The following sections provides the necessary references.

### R.1	Software LED word

The SoftLED option is used on all the CTP boards. The option is described in the software note Software LEDs (SoftLED option) [2]; the note contains the bit allocation of the SOFT_LED word (address H"57"; read only).

### R.2	BusyProbe option

The BusyProbe option is used on the BUSY boards. The option is described in the software note TooBUSY - a tool for monitoring of sub-detectors' BUSY response [2]; the note contains the bit allocation for common control and status words:

- BUSYMAX_DATA (address H"5A"; read only)

- BUSYMINI_DATA (address H"5B"; read only)

- MINIMAX_CLEAR (address H"5D"; dummy write)

- MINIMAX _LIMIT (address H"15E"; write and read)

MINIMAX_SELECT word, write and read:

Address: H"15C"

Data bits	          Content

D[2..0]	Signal selection code (0 to7):

0, 6, 7	All including LTU BUSY

1	Synchronised BUSY 1 input

2	Synchronised BUSY 2 input

3	L1 Message FIFO Nearly Full status

4	L2 Message FIFO Nearly Full status

5	Synchronised sub-detector BUSY

### R.3	LTUvi counters

The implementation of the counters on the LTU board is the same as the scheme used on all the CTP boards; the logic is described in the software note L0 Counters [2]; the note contains the bit allocation for all control and status words:

- COPY_COUNT (address H"75"; dummy write)

- COPY_BUSY (address H"76"; read only)

- COPY_CLEARADD (address H"77"; dummy write)

- COPY_READ (address H"78"; read only)

- CLEAR_COUNTER (address H"16B"; write and read)

### R.3.1	List of LTUvi counters; address allocation in CounterCopy memory

Time counters (Table 1)

Resolution: 0.4µs

| Address | Firmware notation | Signal | Description |
| --- | --- | --- | --- |
| 0 | VCC | Time | Elapsed time |
| 1 | s_in_busy1 | BUSY1 input | Synchronised BUSY1 input |
| 2 | s_in_busy2 | BUSY2 input | Synchronised BUSY2 input |
| 3 | ltu_busy.s_sbusy | Sub-detector BUSY | Synchronised sub-detector BUSY |
| 4 | ltu_busy.s_busy | BUSY | LTU's internal BUSY; all constituents |
| 5 | l1fifo_wr.l1nearly_full | L1 FIFO full | L1 Message FIFO nearly full status |
| 6 | l2fifo_wr.l2nearly_full | L2 FIFO full | L2 Message FIFO nearly full status |
| 7 | bc_error | BC Error | Synchronised BC Error status |

Table 1	LTUvi time counters - CounterCopy memory allocation

Note:	"Elapsed time" timer (address 0) counts continuously; it is intended for real time measurements.

Fast-signal counters (Table 2)

Rate: arbitrary

| Address | Firmware notation | Signal | Description |
| --- | --- | --- | --- |
| 8 | emu_start.start_all | START | Selected emulation START trigger |
| 9 | ltu_busy.s_busy | BUSY pulse | LTU's internal BUSY transition (25ns) |
| 10 | l1fifo_wr.l1nearly_full | L1 FIFO pulse | L1 Message FIFO full transition (25ns) |
| 11 | l2fifo_wr.l2nearly_full | L2 FIFO pulse | L2 Message FIFO full transition (25ns) |
| 12 | ltu_select.any_error | ANY ERROR | ANY ERROR pulse (25ns) |
| 13 | bc_error | BC Error pulse | BC Error status transition (25ns) |
| 14 | minimax.busy_long | Long BUSY | BUSY over threshold (BusyProbe, 25ns) |

Table 2	LTUvi fast-signal counters - CounterCopy memory allocation

Slow-signal counters (Table 3)

Rate: maximum 1 count per 4µs

| Address | Firmware notation | Signal | Description |
| --- | --- | --- | --- |
| 15 | ltu_select.prepulse | PP Request | PP Request (25ns); includes errors |
| 16 | ltu_select.l0 | L0 | L0 output; includes errors |
| 17 | ltu_select.l1_only | L1 only | Selected L1 (25ns); includes errors |
| 18 | ltu_select.l1_strobe | L1 STROBE | Selected L1 STROBE (25ns); includes errors |
| 19 |  | L2a | L2a strobe (25ns); includes errors |
| 20 |  | L2r | L2r strobe (25ns); includes errors |

Table 3	LTUvi slow-signal counters - CounterCopy memory allocation

## N	Definition of new LTUvi words

### N.1	Reset FPGA Phase Locked Loop command

PLL_RESET word, dummy write:

Address: H"6F"

The word resets the BC PLL in the LTU/LTUvi Logic FPGA.

### N.2	TTCvi emulation logic

### N.2.1	General

In the original ALICE TTC partitions, the TTCvi board had been used to control data transmission over the Channel B; the data had been written from the LTU board acting as a VME master; or from the VME processor. The Channel A transmission had been fully controlled by the LTU board.

In the LTUvi version, the generation of the Channel B serial data stream has been "imported" into the LTU; the TTCvi board is no longer required. The LTUvi board is no longer a VME master - the internal Device Wants Bus (DWB) signal is permanently cleared.

In order to make the "transition" as transparent to the software as practical, the TTCvi's approach, the conventions, the word formats, etc. have all been preserved as fully as possible; all the new LTUvi words that are used to control the TTCvi emulator logic are described in the following sections.

The TTC Channel B is used to transmit the following data streams:

| Data stream | Frame format | Priority |
| --- | --- | --- |
| ORBIT | Broadcast frame | 0 |
| PREPULSE | Broadcast frame | 1 |
| L1 Message | Individually-addressed frame | 2 |
| L2 Message | Individually-addressed frame | 3 |
| VME word | Individually-addressed frame | 4 |

The formats of the Broadcast Frame (16 serial bits) and of the Individually-addressed Frame (42 serial bits) are define in [3], Chapter 2.

The data part (8 bits; [3], Chapter 5) of the ORBIT transmission is set by the firmware to H"FC"; in case of the PREPULSE transmission, the data bits are H"01".

Priority level 0 is the highest; lower priority transmissions wait until the higher priority transmissions - in progress or pending - are completed. In order to secure undisturbed timing of the ORBIT and PREPULSE signals, the two transmissions are preceded by an Inhibit interval [4]; during the interval, a start of any new, lower priority transmission is prevented; the widths of the interval is fixed by the firmware to 44 BCs. A correct transmission of the ORBIT is assured by design. In order to secure a correct transmission of the PREPULSE, the requested transmission must not coincide with the ORBIT Inhibit interval; if it does, the PREPULSE transmission shall be abandoned. The timing of the ORBIT and the PREPULSE transmission is scheduled by software settings (ORBIT_TIME and PP_TIME words, see the following sections).

### N.2.2	Transmission of ORBIT and PREPULSE

The transmission is automatic - no software intervention is required; the software only controls the timing - ORBIT_TIME and PP_TIME words.

ORBIT_TIME word, write and read:

Address: H"170"

Data bits	          Content

D[11..0]	BC number that is the start of ORBIT Inhibit interval(0 - 3563)

Note:	Control software must prevent attempts to write values higher than 3563.

The ORBIT Inhibit interval is 44 BCs long (ScopeProbe output A, code 22); it is followed by the broadcast frame transmission (ORBIT transmit, ScopeProbe output B, code 24); at the end of the transmissions (16 BCs), the TTCrx's internal BC Counter is reset.

PP_TIME word, write and read:

Address: H"171"

Data bits	          Content

D[11..0]	BC number that is the start of PP Inhibit interval (0 - 3563)

Note:	Control software must prevent attempts to write values higher than 3563.

The PP Inhibit interval is 44 BCs long (ScopeProbe output A, code 23); it is followed by the broadcast frame transmission (PP transmit, ScopeProbe output B, code 25); at the end of the transmissions (16 BCs), the PP signal can be decoded at the TTCrx output.

Note:	PP transmission shall be abandoned if its start (at the end of PP Inhibit interval - PP_TIME word) is due to occur during the ORBIT Inhibit interval (set by the ORBIT_TIME word).

### N.2.3	TTC data transmission from the VME processor

The VME processor writes data into the TTC FIFO of the LTUvi board (TTC_DATA word); FIFO's capacity is 128 32-bit words; the TTC_STATUS word reads the FIFO's current status. The data transmission has the lowest priority (see the table in N.2.1); the transmission (42 serial bits; Individually-addressed Frame format - [3], Chapter 2) takes place during the VME transmit interval (ScopeProbe output B, code 28).

TTC_DATA word, write and read:

Address: H"172"

Bit allocation:

Data bits		Content

D[31]		Don't care

D[30..17]	TTCrx address (14 bits; all 0: broadcast)

D[16]		E (1: external; 0: internal)

D[15..8]	Sub-address (8 bits)

D[7..0]		Data (8 bits)

The data format is identical to the TTCvi protocol - [4], TTCvi Registers and VME Address Map, Long-format asynchronous cycles; the two 16-bit words $C0 and $C2 are just transmitted as a single 32-bit word.

Note:	The TTC_DATA read word returns the content of the last write access (SpyMemory option).

TTC_STATUS word, read only:

Address: H"72"

Bit allocation:

Data bits		Content

D[0]		TTC FIFO empty

D[1]		TTC FIFO full

### N.3	ScopeProbe signal selection - LTUvi board

The list of signals connected to the ScopeProbe outputs A and B is shown in the following tables; the list corresponds to the firmware version H"BX".The ScopeProbe option is described in [5], section 3.13; signal selection is controlled by the VME word SCOPE_SELECT [1].

Few words of explanation:

- Comment selected means: as received from the CTP in Global mode; or generated by the LTU emulator in Stand-alone mode.

- Comment no errors/includes errors explains whether the displayed signal is taken before or after the introduction of emulated errors.

- Comment no delay/delay, in case of the outputs Channel A and Channel B, explains whether the displayed signal is taken before or after the introduction of programmable ½BC delay.

- Comment not registered indicates that the displayed signal might show decoding glitches.

- Some signals - that are likely to be used as the scope trigger input - are available at both outputs A and B.

### ScopeProbe output A

| Code | Firmware notation | Signal | Description |
| --- | --- | --- | --- |
| 0 | bc | BC | 40MHz LHC clock; includes delay; also at B0 |
| 1 | ltu_select.orbit_sel | ORBIT | Selected ORBIT (1µs pulse); also at B17 |
| 2 | ltu_select.prepulse_sel | PP request | PP request (25ns pulse); no errors |
| 3 | ltu_select.l1_strobe | L1 STROBE | Selected L1 STROBE; includes errors |
| 4 | ltu_select.l2_strobe | L2 STROBE | Selected L2 STROBE; includes errors |
| 5 | in_pulser | PULSER input | PULSER input signal |
| 6 | in_busy1 | BUSY1 input | BUSY 1 input signal |
| 7 | in_busy2 | BUSY2 input | BUSY 2 input signal |
| 8 | !write & strobe | VME read | VME read strobe |
| 9 | write & strobe | VME write | VME write strobe |
| 10 | strobe | VME strobe | VME strobe (read or write) |
| 11 | ttcvi.channel_b | Channel B | TTC Channel B output; no delay |
| 12 | minimax.busy_long | Long BUSY | BusyProbe BUSY_LONG output (25ns pulse); also at B30 |
| 13 | adc_in | ADC input | Input to the ADC (LTU synchronisation) |
| 14 | ltu_select.err_request | Error request | Pending Error Request (Error on Demand option) |
| [15..16] | GND | GND | Not used; connected to ground |
| 17 | emu_st.start_all | START | Selected START signal (emulation trigger) |
| 18 | ltu_select.l0_sel | L0 | Selected L0 (25ns pulse); no errors |
| 19 | vme_clock | VME clock | 16MHz VME clock |
| 20 | ltu_select.l1 | Channel A | TTC Channel A output no delay |
| 21 | ltu_select.l1_sel | L1 only | Selected L1 (25ns pulse); no errors |
| 22 | ttcvi.orbit_inhibit | ORBIT inhibit | ORBIT inhibit interval; 44 BCs; precedes ORBIT transmission (ORBIT select, B24) |
| 23 | ttcvi.pp_inhibit | PP inhibit | PREPULSE inhibit interval; 44 BCs; precedes PREPULSE transmission (PP select, B25) |
| 24 | ttcvi.vmefifo_write | VME TTC write | VME FIFO write strobe (25ns pulse) |
| 25 | ttcvi.ttc_busy | TTC BUSY | TTC Channel B transmission in progress |
| [26..31] | GND | GND | Not used; connected to ground |

### ScopeProbe output B

| Code | Firmware notation | Signal | Description |
| --- | --- | --- | --- |
| 0 | bc | BC | 40MHz LHC clock; includes delay; also at A0 |
| 1 | in_bc | BC input | 40MHz LHC clock; input signal, no delay |
| 2 | out_orbit | Channel B output | TTC Channel B output; includes delay and errors |
| 3 | out_prepulse | PP output | PP request (25ns pulse); includes errors (The output is not used) |
| 4 | out_1l0 | L0 output | L0 output; includes errors |
| 5 | out_l1 | Channel A output | TTC Channel A output; includes delay and errors; L0 and L1 if L0 over TTC option |
| 6 | ltu_select.l1_data | L1 DATA | Selected serial L1 DATA; registered |
| 7 | ltu_select.l2_data | L2 DATA | Selected serial L2 DATA |
| 8 | emu_st.start | Emulation start | Start of the emulation sequence (not registered) |
| 9 | ltu_busy.busy | BUSY | LTU internal BUSY; all constituents |
| 10 | l1fifo_wr.l1fifo_empty | L1FIFO empty | L1 Message FIFO empty status |
| 11 | l2fifo_wr.l2fifo_empty | L2FIFO empty | L2 Message FIFO empty status |
| 12 | l1fifo_wr.l1nearly_full | L1FIFO full | L1 Message FIFO nearly full status |
| 13 | l2fifo_wr.l2nearly_full | L2FIFO full | L2 Message FIFO nearly full status |
| 14 | ltu_select.any_error | Any error | Any error signal (25ns pulse) |
| 15 | emu_lg.l1_strobe | Emu L1 STROBE | L1 STROBE signal from the emulator |
| 16 | emu_lg.l2_data | Emu L2 DATA | Serial L2 DATA from the emulator |
| 17 | ltu_select.orbit_sel | ORBIT | Selected ORBIT (1µs pulse); also at A1 |
| 18 |  | L2a strobe | L2a strobe signal (25ns pulse) |
| 19 |  | L2r strobe | L2r strobe signal (25ns pulse) |
| 20 | bc_error | BC error | BC error flag |
| 21 | ltu_select.l1_only | L1 only | Selected L1 (25ns pulse); includes errors |
| 22 | ltu_select.l1_data_sel | L1 DATA (nr) | Selected serial L1 DATA; not registered |
| 23 | ttcvi.pp_request | PP pending | Pending PREPULSE request |
| 24 | ttcvi.orbit_select | ORBIT transmit | ORBIT transmission in progress (17 BCs) |
| 25 | ttcvi.pp_select | PP transmit | PP transmission in progress (17 BCs) |
| 26 | ttcvi.l1_select | L1 transmit | L1 word transmission in progress (43 BCs) |
| 27 | ttcvi.l2_select | L2 transmit | L2 word transmission in progress (43 BCs) |
| 28 | ttcvi.vme_select | VME transmit | VME TTC transmission in progress (43 BCs) |
| 29 | ttcvi.vmefifo_empty | VME FIFO empty | VME TTC FIFO empty status |
| 30 | minimax.busy_long | Long BUSY | BusyProbe BUSY_LONG output (25ns pulse); also at A12 |
| 31 | GND | GND | Not used; connected to ground |

### N.4	Font panel LEDs - update for the LTUvi board

The logic and the operation of the LTU's front panel LEDs have been fully described in [1], section 3.3. In the LTUvi version of the board, the Spare LED has been attributed a somewhat different meaning:

Spare LED:	on - if Global Mode; off - if Stand-alone Mode

Note:	The LEDs PP, L0 and L1 do not display signals that are "masked" by the error-generating logic.

### N.5	Font panel connections - update for the LTUvi board

Front panel input and output connections of the original LTU have been described in [5], section 3.5.6. The following updates apply to the LTUvi version of the board:

Spare:	ECL output of the BC clock. The clock is a delayed BC input from the TTCex board; it is the main clock of the LTUvi's FPGA.

L1:	Channel A connection to the TTCex board (ECL output); normally, the L1 signal, but, in case of the "L0 over TTC" option, both the L0 (code B"10") and the L1 (code B"11") signals.

ORBIT:	Channel B connection to the TTCex board (ECL output).

### N.6	Cable connections in the TTC partition - LTUvi board

Cable connections in the original ALICE TTC partition are shown in [5], section 3.14.2, Figure 3.14.1; the figure has been reproduced here (Figure A) to ease the comparison. Connections in the new LTUvi partition are shown in Figure B.

Figure A	Connections in the TTC partition (LTU version - old)

Figure B	Connections in the TTC partition (LTUvi version - new)